CHEMICAL STORAGE PLAN FOR DENTAL DEPARTMENT

The following general guidelines apply to the Dental Clinic which use the products listed below to maintain the department and train in dental course work.

CONCETRATED ACID - PURE VAC – is to be stored alone in secondary containment. Special effort must be made in storing and dispensing the liquid due to its very corrosive nature (it will burn the skin). This chemical must always be returned to a secondary containment tray, assuring that the dispenser nozzle points into the tray so that it does not leak elsewhere.  This dispenser must be protected to assure that it does not leak onto the shelf, onto persons or about the clinic.  The total volume of PURA VAC should not exceed 8 containers, with one “IN USE”.  The one in use should be stored beneath the sink (in containment) protecting the dispenser from accidental discharge.

WEAK BASES - can be stored together in the same secondary container. The following items are in the same chemical family and can placed together.

  - Simple Green

  - Windex Glass Cleaner

  - Speed Clean

WEAK ACIDS – can be stored together in the same secondary container.  This group includes most of the disinfectants and “chlorine like” products.  These are to be placed together away from the weak bases.

- “Birex” Disinfectant

- Comet cleaner

- Anti-bacterial soap (Dial or others)

- Bleach

OTHER ITEMS:

- Keep the bleach and the liquid soap and fabric softener used with the clothes washer within 2ndary containment, in separate units.

- Liquids should not be placed above the sink for storage.

- All material dispensed into little bottles should be clearly labeled in English even if the chemical “looks like” what it really is (example Simple Green).  Additionally, these bottle must be kept closed except with in use.